NetIQ Vivinet Assessor

Voice over IP Readiness Assessment

Complete Report

Prepared by:

NetIQ

for
Sample Company

Friday, December 09, 2005 11:40:11 AM

# Table of Contents

Table of Contents	2

Overview	5

VoIP Readiness Summary	5

Network Inventory	6

Routers	6

Switches	6

Links	7

Utilization Assessment	8

What Was Assessed	8

Utilization Readiness Ratings	8

Router Readiness Ratings	9

Switch Readiness Ratings	9

Link Readiness Ratings	9

Utilization Readiness Summary	10

Router Measurements	10

Router Average CPU Utilization	11

Router Average Memory Utilization	11

Router Input Queue Drops	12

Router Output Queue Drops	12

Router Buffer Errors	13

Router CRC Errors	13

Router Measurements by Day	14

Router Average CPU Utilization by Day	14

Router Average Memory Utilization by Day	15

Router Input Queue Drops by Day	15

Router Output Queue Drops by Day	16

Router Buffer Errors by Day	16

Router CRC Errors by Day	17

Router Measurements by Hour	17

Router Average CPU Utilization by Hour	18

Router Average Memory Utilization by Hour	18

Router Input Queue Drops by Hour	19

Router Output Queue Drops by Hour	19

Router Buffer Errors by Hour	20

Router CRC Errors by Hour	20

Router Utilization Details	21

Switch Measurements	22

Switch Average Backplane Utilization	22

Switch Average CPU Utilization	23

Switch Measurements by Day	23

Switch Average Backplane Utilization by Day	24

Switch Average CPU Utilization by Day	24

Switch Measurements by Hour	25

Switch Average Backplane Utilization by Hour	25

Switch Average CPU Utilization by Hour	26

Switch Utilization Details	26

Link Average Bandwidth Utilization	28

Link Average Bandwidth Utilization by Day	28

Link Average Bandwidth Utilization by Hour	29

Link Utilization Details	29

Bandwidth Modeling Results	31

VoIP Quality Assessment	32

What Was Assessed	32

Call Quality Ratings	32

Potential Lost Revenue	33

Call Summary	34

Call Quality Summary	34

Factors Affecting Call Quality	34

Calls by Group	35

Call Quality Summary by Call Group	35

Call Quality Evaluation by Call Group	35

Factors Affecting Call Quality by Call Group	36

Delay Summary by Call Group	37

Delay Evaluation by Call Group	37

Jitter Summary by Call Group	38

Jitter Evaluation by Call Group	38

Lost Data Summary by Call Group	39

Lost Data Evaluation by Call Group	39

Calls by Day	40

Call Quality Evaluation by Day	40

Factors Affecting Call Quality by Day	40

Delay Evaluation by Day	41

Jitter Evaluation by Day	41

Lost Data Evaluation by Day	42

Calls by Hour	43

Call Quality Evaluation by Hour	43

Factors Affecting Call Quality by Hour	43

Delay Evaluation by Hour	44

Jitter Evaluation by Hour	44

Lost Data Evaluation by Hour	45

Calls by Call Script	46

Call Quality Summary by Call Script	46

Call Quality Evaluation by Call Script	46

Factors Affecting Call Quality by Call Script	47

Availability Analysis	48

Explanation	48

Availability Summary	48

Availability by Day	48

Availability by Hour	49

All Groups Call Quality Summary	50

Appendix A – VoIP Quality Configuration	51

Call Groups	51

Background Traffic	52

Endpoints	52

Call Scripts	52

Assessment Options	53

VoIP Quality - Design	54

Appendix B – Call Quality Score Calculation	55

Appendix C – Codec Types and Values	56

# Overview

This report summarizes the results of a VoIP Readiness Assessment performed by NetIQ Vivinet Assessor.  A VoIP Readiness Assessment determines whether a particular network is ready to handle voice over IP (VoIP) in its present configuration.  During the assessment, Vivinet Assessor performed some or all of the following tasks:

- Compiled an inventory of devices and links on the network.

- Analyzed the configuration of devices discovered during the inventory.

- Monitored the utilization of selected devices and links.

- Modeled link capacity based on the bandwidth required for VoIP calls.

- Evaluated the quality of simulated VoIP calls.

Results from any or all of these tasks are presented in this VoIP Readiness Assessment report.

# VoIP Readiness Summary

This report contains information gathered during the VoIP Readiness Assessment described below:

| Assessment Name | Sample |
| --- | --- |
| Network Inventory Status | Discovery complete. |
| Configuration Assessment Status | No data available. |
| Utilization Assessment Status | Monitoring complete. |
| VoIP Quality Assessment Status | Assessment complete. |
| Assessment Report Created Using | NetIQ Vivinet Assessor 3.2, build level 7138 |
| Time Report Generated | Friday, December 09, 2005 11:40:11 AM |
| Report Filter | None |
| Content Filter: Sections Omitted | Call Group Details Appendix - Router Details Appendix - Router Interface Details Appendix - Switch Details Appendix - Link Details |

# Network Inventory

Vivinet Assessor discovered routers, switches, and links on the network and queried them to gather information about their suitability for voice over IP traffic.  Refer to the appendices, if included in the report, for detailed information about each discovered device and link that was selected for detailed reporting.  The following tables summarize the findings:

|  | # Discovered | Details Included For |
| --- | --- | --- |
| Routers | 4 | 4 |
| Switches | 12 | 12 |
| Links | 4 | 4 |

## Routers

Of 4 routers that were discovered during the Network Inventory, 4 routers were selected for detailed reporting.  The table below identifies each selected router.  Refer to the appendices, if included in the report, for detailed information about each selected router.

| Name | Location | Contact Address | Vendor | Model | OS Revision |
| --- | --- | --- | --- | --- | --- |
| beloverly01.bellevue.netiq | Bellevue | 10.140.129.24 | Cisco | cisco3640 | 12.2(4)T1 |
| houhicksj01.houston.netiq | Houston | 10.152.127.46 | Wellfleet | wfSwSeries7 | 15.4.0.1 |
| ralddozerjs01.raleigh.netiq | Raleigh | 10.0.60.165 | Rapid City Communication | rcA8610 | 3.3.0.0 |
| sjcdhandst01.sanjose.netiq | San Jose | 10.96.6.229 | Cisco | cisco4700 | 12.1(9) |

## Switches

Of 12 switches that were discovered during the Network Inventory, 12 switches were selected for detailed reporting.  The table below identifies each selected switch.  Refer to the appendices, if included in the report, for detailed information about each selected switch.

| Name | Location | Contact Address | Vendor | Model | OS Revision | Port Count |
| --- | --- | --- | --- | --- | --- | --- |
| bellevue-rack1.bellevue.netiq | Bellevue | 10.198.148.223 | Cisco | catalyst2924MXL | 11.2(9.0) | 24 |
| bellevue-rack2.bellevue.netiq | Bellevue | 10.123.135.243 | Cisco | catalyst3524XL | 12.0(5.2)XU | 24 |
| bellevue-rack3.bellevue.netiq | Bellevue | 10.222.223.48 | SynOptics | sreg-BayStack350-24T-ethSwitchNMM | 2.1.1 | 24 |
| houston-rack1.houston.netiq | Houston | 10.95.230.183 | Extreme Networks | alpine3804 | 6.2.1b20 | 48 |
| ral-rack1.raleigh.netiq | Raleigh | 10.78.9.130 | Cisco | catalyst2924MXL | 11.2(8.6)SA6 | 24 |
| ral-rack2.raleigh.netiq | Raleigh | 10.177.35.53 | Cisco | wsc4006sysID | 11.2(8.8)SA4 | 98 |
| ral-rack3.raleigh.netiq | Raleigh | 10.240.249.66 | Cisco | catalyst295012G | 12.1(6)EA2a | 12 |
| sanjose-rack1.sanjose.netiq | San Jose | 10.128.114.92 | Cisco | catalyst3524XL | 12.0(5.2)XU | 24 |
| sanjose-rack2.sanjose.netiq | San Jose | 10.227.174.151 | SynOptics | sreg-BayStack350-24T-ethSwitchNMM | 2.1.1 | 24 |
| sanjose-rack3.sanjose.netiq | San Jose | 10.120.126.207 | Alcatel | deviceOmniSwitch6624 | 5.1.2.28.R02 | 24 |
| sanjose-rack4.sanjose.netiq | San Jose | 10.88.63.210 | Cisco | catalyst2908xl | 11.2(8.6)SA6 | 8 |
| sanjose-rack5.sanjose.netiq | San Jose | 10.19.96.17 | Cisco | wsc5000sysID | 11.2(8.8)SA4 | 16 |

## Links

Of 4 links that were discovered during the Network Inventory, 4 links were selected for detailed reporting.  The table below identifies each selected link.  Refer to the appendices, if included in the report, for detailed information about each selected link.

| Name | Type | Speed (kbps) | CIR (kbps) |
| --- | --- | --- | --- |
| beloverly01.bellevue.netiq:FastEthernet0/0 - sjcdhandst01.sanjose.netiq:FastEthernet0/0 | ethernetCsmacd(6) | 100000 | n/a |
| sjcdhandst01.sanjose.netiq:FastEthernet0/0 - ralddozerjs01.raleigh.netiq:FastEthernet0/0 | ethernetCsmacd(6) | 100000 | n/a |
| sjcdhandst01.sanjose.netiq:Serial1:95 - Unknown | frameRelay(32) | 1544 | 512 |
| sjcdhandst01.sanjose.netiq:Serial2 - beloverly01.bellevue.netiq:Serial2 | propPointToPointSerial(22) | 2048 | n/a |

# Utilization Assessment

Vivinet Assessor monitored the routers, switches, and links discovered on the network to gather information about their current utilization and determine their capacity for carrying VoIP traffic.  From SNMP polling, Vivinet Assessor collected device-specific information about queuing and queue drops; link bandwidth utilization; CPU and memory utilization; and errors.  A device readiness rating was assigned to each monitored device or link based on the findings.

## What Was Assessed

Vivinet Assessor conducted the following Utilization Assessment:

| Assessment Name | Sample |
| --- | --- |
| Utilization Assessment Status | Monitoring complete. |
| Utilization Assessment Length | 2 days |
| Start Time | Monday, October 17, 2005 10:00:00 AM |
| Scheduled Stop Time | Wednesday, October 19, 2005 10:00:00 AM |
| Actual Stop Time | Wednesday, October 19, 2005 10:00:00 AM |
| Time Report Generated | Friday, December 09, 2005 11:40:11 AM |
| Report Filter | None |

Every 5 minutes, device Management Information Bases (MIBs) were polled and their utilization statistics were recorded.

### Utilization Readiness Ratings

Readiness to carry VoIP was assessed based on the percentage of all measurements for monitored devices and links that fell into each readiness ratings category.  Ratings were assigned according to the following result ranges for this assessment:

| Rating | Indicator | Meaning |
| --- | --- | --- |
| Good |  | At least 99.00% of collected measurements rated Good. |
| Acceptable |  | At least 98.00% of collected measurements rated Good or Acceptable. Reconfiguration or an upgrade may be necessary for VoIP readiness. |
| Poor |  | Any lower value. Too few measurements were within acceptable parameters for this assessment. The device or link may not be ready to carry high-quality VoIP traffic. |
| Unavailable | n/a | No results were available to report. |

### Router Readiness Ratings

The table below shows how router measurements were rated.  Ratings were based on the router result ranges for this assessment:

| Measurement | Good | Acceptable | Poor |
| --- | --- | --- | --- |
| Average CPU Utilization (%) | Less than or equal to 30.0 | Less than or equal to 50.0 | Any higher value |
| Average Memory Utilization (%) | Less than or equal to 30.0 | Less than or equal to 50.0 | Any higher value |
| Input Queue Drops (%) | Less than or equal to 2.0 | Less than or equal to 8.0 | Any higher value |
| Output Queue Drops (%) | Less than or equal to 2.0 | Less than or equal to 8.0 | Any higher value |
| Buffer Errors | Less than or equal to 0 | Less than or equal to 1 | Any higher value |
| CRC Errors (%) | Less than or equal to 2.0 | Less than or equal to 8.0 | Any higher value |

### Switch Readiness Ratings

The table below shows how switch measurements were rated.  Ratings were based on the switch result ranges for this assessment:

| Measurement | Good | Acceptable | Poor |
| --- | --- | --- | --- |
| Average Backplane Utilization (%) | Less than or equal to 50.0 | Less than or equal to 75.0 | Any higher value |
| Average CPU Utilization (%) | Less than or equal to 30.0 | Less than or equal to 50.0 | Any higher value |

### Link Readiness Ratings

The table below shows how link measurements were rated.  Ratings were based on the link result ranges for this assessment:

| Measurement | Good | Acceptable | Poor |
| --- | --- | --- | --- |
| Average Bandwidth Utilization (%) | Less than or equal to 30.0 | Less than or equal to 50.0 | Any higher value |

## Utilization Readiness Summary

The chart below shows the percentage of all monitored devices and links that fell into each category of device readiness.

## Router Measurements

The chart below shows the percentage of all measurements collected for monitored routers that fell into each category of device readiness.

### Router Average CPU Utilization

The chart below shows the percentage of all Average CPU Utilization measurements collected for monitored routers that fell into each category of device readiness.

### Router Average Memory Utilization

The chart below shows the percentage of all Average Memory Utilization measurements collected for monitored routers that fell into each category of device readiness.

### Router Input Queue Drops

The chart below shows the percentage of all Input Queue Drops measurements collected for monitored routers that fell into each category of device readiness.

### Router Output Queue Drops

The chart below shows the percentage of all Output Queue Drops measurements collected for monitored routers that fell into each category of device readiness.

### Router Buffer Errors

The chart below shows the percentage of all Buffer Errors measurements collected for monitored routers that fell into each category of device readiness.

### Router CRC Errors

The chart below shows the percentage of all CRC Errors measurements collected for monitored routers that fell into each category of device readiness.

## Router Measurements by Day

The chart below breaks out router device readiness per day by evaluating all measurements collected for monitored routers according to the result ranges defined for the assessment.

### Router Average CPU Utilization by Day

The chart below evaluates router Average CPU Utilization values for each day of the assessment according to the router Average CPU Utilization result ranges defined for the assessment.

### Router Average Memory Utilization by Day

The chart below evaluates router Average Memory Utilization values for each day of the assessment according to the router Average Memory Utilization result ranges defined for the assessment.

### Router Input Queue Drops by Day

The chart below evaluates router Input Queue Drops values for each day of the assessment according to the router Input Queue Drops result ranges defined for the assessment.

### Router Output Queue Drops by Day

The chart below evaluates router Output Queue Drops values for each day of the assessment according to the router Output Queue Drops result ranges defined for the assessment.

### Router Buffer Errors by Day

The chart below evaluates router Buffer Errors values for each day of the assessment according to the router Buffer Errors result ranges defined for the assessment.

### Router CRC Errors by Day

The chart below evaluates router CRC Errors values for each day of the assessment according to the router CRC Errors result ranges defined for the assessment.

## Router Measurements by Hour

The chart below breaks out router device readiness per hour by evaluating all measurements collected for monitored routers according to the result ranges defined for the assessment.

### Router Average CPU Utilization by Hour

The chart below evaluates router Average CPU Utilization values by hour of the day according to the router Average CPU Utilization result ranges defined for the assessment. Each bar shows Average CPU Utilization values averaged across all days of the assessment for that hour.

### Router Average Memory Utilization by Hour

The chart below evaluates router Average Memory Utilization values by hour of the day according to the router Average Memory Utilization result ranges defined for the assessment. Each bar shows Average Memory Utilization values averaged across all days of the assessment for that hour.

### Router Input Queue Drops by Hour

The chart below evaluates router Input Queue Drops values by hour of the day according to the router Input Queue Drops result ranges defined for the assessment. Each bar shows Input Queue Drops values averaged across all days of the assessment for that hour.

### Router Output Queue Drops by Hour

The chart below evaluates router Output Queue Drops values by hour of the day according to the router Output Queue Drops result ranges defined for the assessment. Each bar shows Output Queue Drops values averaged across all days of the assessment for that hour.

### Router Buffer Errors by Hour

The chart below evaluates router Buffer Errors values by hour of the day according to the router Buffer Errors result ranges defined for the assessment. Each bar shows Buffer Errors values averaged across all days of the assessment for that hour.

### Router CRC Errors by Hour

The chart below evaluates router CRC Errors values by hour of the day according to the router CRC Errors result ranges defined for the assessment. Each bar shows CRC Errors values averaged across all days of the assessment for that hour.

## Router Utilization Details

The table below summarizes the utilization measurements collected for each router monitored during the assessment.  Readiness Ratings indicators show the rating for each measurement.

| Name | Results | Measurements (% of time) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | All | Average CPU Utilization | Average Memory Utilization | Input Queue Drops | Output Queue Drops | Buffer Errors | CRC Errors |
| beloverly01.bellevue.netiq | Rating |  |  |  |  |  |  |  |
|  | Good | 100.00% | 100.00% | 100.00% | 100.00% | 100.00% | 100.00% | 100.00% |
|  | Acceptable | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
|  | Poor | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
|  | Unavailable | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
|  | Average | n/a | 10.0% | 9.9% | 0.2% | 0.2% | 0 | 0.2% |
| houhicksj01.houston.netiq | Rating |  |  |  |  |  |  |  |
|  | Good | 100.00% | 100.00% | 100.00% | 100.00% | 100.00% | 100.00% | 100.00% |
|  | Acceptable | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
|  | Poor | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
|  | Unavailable | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
|  | Average | n/a | 9.7% | 9.5% | 0.2% | 0.2% | 0 | 0.2% |
| ralddozerjs01.raleigh.netiq | Rating |  |  |  |  |  |  |  |
|  | Good | 84.58% | 77.30% | 88.73% | 73.14% | 72.10% | 98.27% | 97.92% |
|  | Acceptable | 13.78% | 17.68% | 9.71% | 25.30% | 26.17% | 1.73% | 2.08% |
|  | Poor | 1.65% | 5.03% | 1.56% | 1.56% | 1.73% | 0.00% | 0.00% |
|  | Unavailable | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
|  | Average | n/a | 17.3% | 12.8% | 1.6% | 1.7% | 0 | 0.3% |
| sjcdhandst01.sanjose.netiq | Rating |  |  |  |  |  |  |  |
|  | Good | 84.52% | 77.30% | 88.04% | 73.83% | 70.88% | 98.61% | 98.44% |
|  | Acceptable | 13.92% | 19.41% | 9.53% | 23.74% | 27.90% | 1.39% | 1.56% |
|  | Poor | 1.56% | 3.29% | 2.43% | 2.43% | 1.21% | 0.00% | 0.00% |
|  | Unavailable | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
|  | Average | n/a | 16.5% | 13.3% | 1.6% | 1.8% | 0 | 0.3% |

## Switch Measurements

The chart below shows the percentage of all measurements collected for monitored switches that fell into each category of device readiness.

### Switch Average Backplane Utilization

The chart below shows the percentage of all Average Backplane Utilization measurements collected for monitored switches that fell into each category of device readiness.

### Switch Average CPU Utilization

The chart below shows the percentage of all Average CPU Utilization measurements collected for monitored switches that fell into each category of device readiness.

## Switch Measurements by Day

The chart below breaks out switch device readiness per day by evaluating all measurements collected for monitored switches according to the result ranges defined for the assessment.

### Switch Average Backplane Utilization by Day

The chart below evaluates switch Average Backplane Utilization values for each day of the assessment according to the switch Average Backplane Utilization result ranges defined for the assessment.

### Switch Average CPU Utilization by Day

The chart below evaluates switch Average CPU Utilization values for each day of the assessment according to the switch Average CPU Utilization result ranges defined for the assessment.

## Switch Measurements by Hour

The chart below breaks out switch device readiness per hour by evaluating all measurements collected for monitored switches according to the result ranges defined for the assessment.

### Switch Average Backplane Utilization by Hour

The chart below evaluates switch Average Backplane Utilization values by hour of the day according to the switch Average Backplane Utilization result ranges defined for the assessment. Each bar shows Average Backplane Utilization values averaged across all days of the assessment for that hour.

### Switch Average CPU Utilization by Hour

The chart below evaluates switch Average CPU Utilization values by hour of the day according to the switch Average CPU Utilization result ranges defined for the assessment. Each bar shows Average CPU Utilization values averaged across all days of the assessment for that hour.

## Switch Utilization Details

The table below summarizes the utilization measurements collected for each switch monitored during the assessment.  Readiness Ratings indicators show the rating for each measurement.

| Name | Location | Model | OS Revision | Results | Measurements (% of time) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | All | Average Backplane Utilization | Average CPU Utilization |
| bellevue-rack1.bellevue.netiq | Bellevue | catalyst2924MXL | 11.2(9.0) | Rating |  |  |  |
|  |  |  |  | Good | 100.00% | 100.00% | 100.00% |
|  |  |  |  | Acceptable | 0.00% | 0.00% | 0.00% |
|  |  |  |  | Poor | 0.00% | 0.00% | 0.00% |
|  |  |  |  | Unavailable | 0.00% | 0.00% | 0.00% |
|  |  |  |  | Average | n/a | 10.7% | 9.7% |
| bellevue-rack2.bellevue.netiq | Bellevue | catalyst3524XL | 12.0(5.2)XU | Rating |  |  |  |
|  |  |  |  | Good | 88.47% | 93.07% | 83.88% |
|  |  |  |  | Acceptable | 11.44% | 6.93% | 15.94% |
|  |  |  |  | Poor | 0.09% | 0.00% | 0.17% |
|  |  |  |  | Unavailable | 0.00% | 0.00% | 0.00% |
|  |  |  |  | Average | n/a | 25.4% | 16.1% |
| bellevue-rack3.bellevue.netiq | Bellevue | sreg-BayStack350-24T-ethSwitchNMM | 2.1.1 | Rating |  |  |  |
|  |  |  |  | Good | 88.56% | 93.93% | 83.19% |
|  |  |  |  | Acceptable | 11.44% | 6.07% | 16.81% |
|  |  |  |  | Poor | 0.00% | 0.00% | 0.00% |
|  |  |  |  | Unavailable | 0.00% | 0.00% | 0.00% |
|  |  |  |  | Average | n/a | 25.8% | 15.9% |
| houston-rack1.houston.netiq | Houston | alpine3804 | 6.2.1b20 | Rating |  |  |  |
|  |  |  |  | Good | 89.69% | 93.24% | 86.14% |
|  |  |  |  | Acceptable | 10.23% | 6.59% | 13.86% |
|  |  |  |  | Poor | 0.09% | 0.17% | 0.00% |
|  |  |  |  | Unavailable | 0.00% | 0.00% | 0.00% |
|  |  |  |  | Average | n/a | 25.7% | 15.7% |
| ral-rack1.raleigh.netiq | Raleigh | catalyst2924MXL | 11.2(8.6)SA6 | Rating |  |  |  |
|  |  |  |  | Good | 100.00% | 100.00% | 100.00% |
|  |  |  |  | Acceptable | 0.00% | 0.00% | 0.00% |
|  |  |  |  | Poor | 0.00% | 0.00% | 0.00% |
|  |  |  |  | Unavailable | 0.00% | 0.00% | 0.00% |
|  |  |  |  | Average | n/a | 10.8% | 9.5% |
| ral-rack2.raleigh.netiq | Raleigh | wsc4006sysID | 11.2(8.8)SA4 | Rating |  |  |  |
|  |  |  |  | Good | 89.69% | 95.15% | 84.23% |
|  |  |  |  | Acceptable | 10.14% | 4.85% | 15.42% |
|  |  |  |  | Poor | 0.17% | 0.00% | 0.35% |
|  |  |  |  | Unavailable | 0.00% | 0.00% | 0.00% |
|  |  |  |  | Average | n/a | 25.2% | 16.0% |
| ral-rack3.raleigh.netiq | Raleigh | catalyst295012G | 12.1(6)EA2a | Rating |  |  |  |
|  |  |  |  | Good | 88.56% | 95.15% | 81.98% |
|  |  |  |  | Acceptable | 11.18% | 4.85% | 17.50% |
|  |  |  |  | Poor | 0.26% | 0.00% | 0.52% |
|  |  |  |  | Unavailable | 0.00% | 0.00% | 0.00% |
|  |  |  |  | Average | n/a | 25.0% | 16.5% |
| sanjose-rack1.sanjose.netiq | San Jose | catalyst3524XL | 12.0(5.2)XU | Rating |  |  |  |
|  |  |  |  | Good | 100.00% | 100.00% | 100.00% |
|  |  |  |  | Acceptable | 0.00% | 0.00% | 0.00% |
|  |  |  |  | Poor | 0.00% | 0.00% | 0.00% |
|  |  |  |  | Unavailable | 0.00% | 0.00% | 0.00% |
|  |  |  |  | Average | n/a | 11.0% | 9.5% |
| sanjose-rack2.sanjose.netiq | San Jose | sreg-BayStack350-24T-ethSwitchNMM | 2.1.1 | Rating |  |  |  |
|  |  |  |  | Good | 88.73% | 94.11% | 83.36% |
|  |  |  |  | Acceptable | 10.83% | 5.89% | 15.77% |
|  |  |  |  | Poor | 0.43% | 0.00% | 0.87% |
|  |  |  |  | Unavailable | 0.00% | 0.00% | 0.00% |
|  |  |  |  | Average | n/a | 25.2% | 16.1% |
| sanjose-rack4.sanjose.netiq | San Jose | catalyst2908xl | 11.2(8.6)SA6 | Rating |  |  |  |
|  |  |  |  | Good | 89.60% | 94.11% | 85.10% |
|  |  |  |  | Acceptable | 10.31% | 5.89% | 14.73% |
|  |  |  |  | Poor | 0.09% | 0.00% | 0.17% |
|  |  |  |  | Unavailable | 0.00% | 0.00% | 0.00% |
|  |  |  |  | Average | n/a | 25.5% | 15.4% |
| sanjose-rack5.sanjose.netiq | San Jose | wsc5000sysID | 11.2(8.8)SA4 | Rating |  |  |  |
|  |  |  |  | Good | 88.91% | 93.76% | 84.06% |
|  |  |  |  | Acceptable | 11.09% | 6.24% | 15.94% |
|  |  |  |  | Poor | 0.00% | 0.00% | 0.00% |
|  |  |  |  | Unavailable | 0.00% | 0.00% | 0.00% |
|  |  |  |  | Average | n/a | 25.4% | 16.1% |

## Link Average Bandwidth Utilization

The chart below shows the percentage of all Average Bandwidth Utilization measurements collected for monitored switches that fell into each category of device readiness.

## Link Average Bandwidth Utilization by Day

The chart below evaluates link Average Bandwidth Utilization values for each day of the assessment according to the link Average Bandwidth Utilization result ranges defined for the assessment.

## Link Average Bandwidth Utilization by Hour

The chart below evaluates link Average Bandwidth Utilization values by hour of the day according to the link Average Bandwidth Utilization result ranges defined for the assessment. Each bar shows Average Bandwidth Utilization values averaged across all days of the assessment for that hour.

## Link Utilization Details

The table below summarizes the utilization measurements collected for each link monitored during the assessment.  Readiness Ratings indicators show the rating for each measurement.

| Name | Type | Speed (kbps) | CIR (kbps) | Results | Average Bandwidth Utilization |
| --- | --- | --- | --- | --- | --- |
| beloverly01.bellevue.netiq:FastEthernet0/0 - sjcdhandst01.sanjose.netiq:FastEthernet0/0 | ethernetCsmacd(6) | 100000 | n/a | Rating |  |
|  |  |  |  | Good | 97.05% |
|  |  |  |  | Acceptable | 2.25% |
|  |  |  |  | Poor | 0.69% |
|  |  |  |  | Unavailable | 0.00% |
|  |  |  |  | Average | 9.9% |
| sjcdhandst01.sanjose.netiq:FastEthernet0/0 - ralddozerjs01.raleigh.netiq:FastEthernet0/0 | ethernetCsmacd(6) | 100000 | n/a | Rating |  |
|  |  |  |  | Good | 87.69% |
|  |  |  |  | Acceptable | 11.27% |
|  |  |  |  | Poor | 1.04% |
|  |  |  |  | Unavailable | 0.00% |
|  |  |  |  | Average | 14.8% |
| sjcdhandst01.sanjose.netiq:Serial1:95 - Unknown | frameRelay(32) | 1544 | 512 | Rating |  |
|  |  |  |  | Good | 98.44% |
|  |  |  |  | Acceptable | 0.69% |
|  |  |  |  | Poor | 0.87% |
|  |  |  |  | Unavailable | 0.00% |
|  |  |  |  | Average | 7.3% |
| sjcdhandst01.sanjose.netiq:Serial2 - beloverly01.bellevue.netiq:Serial2 | propPointToPointSerial(22) | 2048 | n/a | Rating |  |
|  |  |  |  | Good | 90.64% |
|  |  |  |  | Acceptable | 8.49% |
|  |  |  |  | Poor | 0.87% |
|  |  |  |  | Unavailable | 0.00% |
|  |  |  |  | Average | 13.0% |

# Bandwidth Modeling Results

Based on link capacity and utilization data, Vivinet Assessor constructed models of bandwidth usage for selected links under different VoIP implementation scenarios.  Of 3 links that were modeled, 3 modeled links were selected for detailed reporting.  The following table summarizes the findings.

| Name | sjcdhandst01.sanjose.netiq:FastEthernet0/0 - ralddozerjs01.raleigh.netiq:FastEthernet0/0 | sjcdhandst01.sanjose.netiq:Serial1:95 - Unknown | sjcdhandst01.sanjose.netiq:Serial1:95 - Unknown |
| --- | --- | --- | --- |
| Type | ethernetCsmacd(6) | frameRelay(32) | frameRelay(32) |
| Call Script Name | G.729 (8 kbps) | G.729 (8 kbps) | G.729 (8 kbps) |
| Compress RTP | No | Yes | No |
| Modeled Calls | 10 | 10 | 10 |
| Erlangs | n/a | n/a | n/a |
| Blocking | n/a | n/a | n/a |
| Speed | 100000 kbps | 512 kbps | 512 kbps |
| Average Utilization | 14802.4 kbps | 37.4 kbps | 37.4 kbps |
| Added Call Bandwidth | 312.0 kbps | 124.0 kbps | 268.0 kbps |
| Target Utilization | 75000 kbps | 384 kbps | 384 kbps |
| Above Target | 0.0% | 0.9% | 1.6% |
| Call Capacity | 1929 | 27 | 12 |

# VoIP Quality Assessment

Vivinet Assessor determines how well VoIP will perform once it’s deployed by assessing the quality of simulated VoIP calls sent over the network.  Call quality is evaluated based on the average Mean Opinion Score (MOS) assigned to the simulated calls.  Vivinet Assessor’s objective estimation of a MOS takes into account important factors, such as codec, delay, lost data, and jitter buffer loss, that affect call quality in any VoIP implementation.  A higher MOS estimate indicates higher call quality.

## What Was Assessed

Vivinet Assessor conducted the following VoIP Quality Assessment:

| Assessment Name | Sample |
| --- | --- |
| Schedule Type | Series of scheduled calls. |
| VoIP Quality Assessment Status | Assessment complete. |
| VoIP Quality Assessment Length | 2 days |
| Start Time | Wednesday, October 19, 2005 11:15:00 AM |
| Scheduled Stop Time | Friday, October 21, 2005 11:14:00 AM |
| Actual Stop Time | Friday, October 21, 2005 11:14:00 AM |
| Time Report Generated | Friday, December 09, 2005 11:40:11 AM |
| Report Filter | None |

Every 60 minutes, 24 VoIP calls were emulated; each lasted 1 minute.  Of 6 call groups that were assessed, 6 call groups were selected for detailed reporting.

### Call Quality Ratings

A MOS was calculated for each simulated call.  The values were categorized as shown in the following table:

| Measurement | Good | Acceptable | Poor |
| --- | --- | --- | --- |
| MOS | At least 4.03 | At least 3.60 | Any lower value |
| Delay (ms) | Less than or equal to 150 | Less than or equal to 400 | Any higher value |
| Jitter Buffer Loss (%) | Less than or equal to 0.50 | Less than or equal to 1.00 | Any higher value |
| Lost Data (%) | Less than or equal to 0.50 | Less than or equal to 1.00 | Any higher value |

## Potential Lost Revenue

Telephone service downtime can cause revenue loss, which should be factored into the cost justification for network upgrades to support VoIP.  The VoIP Quality assessment measured downtime as the percentage of simulated VoIP calls whose quality was rated “Poor” or that could not be completed (“Unavailable” calls).  The downtime measurement was then used to estimate revenue loss, based on 100 employees making VoIP calls on the network and 23.78% “Poor” and “Unavailable” calls measured during the assessment.  If no changes are made to the network, Vivinet Assessor projected the following industry-specific revenue loss, shown in the following table:

| Industry | Lost Revenue Per Hour Per Employee | Total Lost Revenue |  |  |
| --- | --- | --- | --- | --- |
|  |  | Per Hour | Per Day | Per Week |
| Financial | $42.67 | $1,014.69 | $8,117.54 | $40,587.70 |

## Call Summary

### Call Quality Summary

The chart below shows the percentage of all calls completed during the assessment that fell into each category of call quality.  "Unavailable" call quality indicates that a call could not be connected or failed.

| Good - At least 4.03 MOS | Acceptable - At least 3.60 MOS | Poor - Any lower value |
| --- | --- | --- |

### Factors Affecting Call Quality

The chart below shows each call quality impairment factor's contribution to the overall call quality.  Each factor's contribution is shown as a percentage of all call-quality impairments.

## Calls by Group

### Call Quality Summary by Call Group

The chart below shows the call quality values for up to 10 call groups, ranked from highest MOS to lowest.

### Call Quality Evaluation by Call Group

The chart below shows the average MOS values for up to 10 call groups, indicating the percentage of completed calls that fell into the MOS result ranges defined for the assessment.

### Factors Affecting Call Quality by Call Group

The chart below breaks out call quality impairment factors to show the impact of each (in points subtracted from the MOS) on the overall call quality for up to 10 call groups with the lowest MOS values.

### Delay Summary by Call Group

The chart below shows the average delay values for up to 10 call groups with the highest delay.

### Delay Evaluation by Call Group

The chart below shows the average delay values for up to 10 call groups, indicating the percentage of completed calls that fell into the delay result ranges defined for the assessment.

### Jitter Summary by Call Group

The chart below shows the average jitter buffer loss values (as a percentage of all datagrams sent) for up to 10 call groups with the highest jitter.  Average jitter (in ms) is also reported separately.

### Jitter Evaluation by Call Group

The chart below shows the average jitter buffer loss values for up to 10 call groups, indicating the percentage of completed calls that fell into the jitter result ranges defined for the assessment.

### Lost Data Summary by Call Group

The chart below shows the average lost data values for up to 10 call groups with the most lost data.

### Lost Data Evaluation by Call Group

The chart below shows the average lost data values for up to 10 call groups, indicating the percentage of completed calls that fell into the lost data result ranges defined for the assessment.

## Calls by Day

### Call Quality Evaluation by Day

The chart below shows each day's average MOS values with a line graph.  The bar graph evaluates each day's MOS values according to the MOS result ranges defined for the assessment and shows the percentage of Good, Acceptable, Poor, and Unavailable calls.

### Factors Affecting Call Quality by Day

The chart below breaks out call quality impairment factors to show the impact of each impairment (in points subtracted from the MOS) on the overall call quality per day.

### Delay Evaluation by Day

The chart below shows each day's average delay values with a line graph.  The bar graph evaluates each day's delay values according to the delay result ranges defined for the assessment and shows the percentage of Good, Acceptable, Poor, and Unavailable calls.

### Jitter Evaluation by Day

The chart below shows each day's average jitter buffer loss values with a line graph.  The bar graph evaluates each day's jitter buffer loss values according to the jitter result ranges defined for the assessment and shows the percentage of Good, Acceptable, Poor, and Unavailable calls.

### Lost Data Evaluation by Day

The chart below shows each day's average lost data values with a line graph.  The bar graph evaluates each day's lost data values according to the lost data result ranges defined for the assessment and shows the percentage of Good, Acceptable, Poor, and Unavailable calls.

## Calls by Hour

### Call Quality Evaluation by Hour

The chart below shows each hour's average MOS values with a line graph.  The bar graph evaluates each hour's MOS values according to the MOS result ranges defined for the assessment and shows the percentage of Good, Acceptable, Poor, and Unavailable calls.

### Factors Affecting Call Quality by Hour

The chart below breaks out call quality impairment factors to show the impact of each impairment (in points subtracted from the MOS) on the overall call quality per hour.

### Delay Evaluation by Hour

The chart below shows each hour's average delay values with a line graph.  The bar graph evaluates each hour's delay values according to the delay result ranges defined for the assessment and shows the percentage of Good, Acceptable, Poor, and Unavailable calls.

### Jitter Evaluation by Hour

The chart below shows each hour's average jitter buffer loss values with a line graph.  The bar graph evaluates each hour's jitter buffer loss values according to the jitter result ranges defined for the assessment and shows the percentage of Good, Acceptable, Poor, and Unavailable calls.

### Lost Data Evaluation by Hour

The chart below shows each hour's average lost data values with a line graph.  The bar graph evaluates each hour's lost data values according to the lost data result ranges defined for the assessment and shows the percentage of Good, Acceptable, Poor, and Unavailable calls.

## Calls by Call Script

### Call Quality Summary by Call Script

The chart below shows the average MOS values for up to 10 call scripts with the lowest MOS.

### Call Quality Evaluation by Call Script

The chart below shows the average MOS values for up to 10 call scripts, indicating the percentage of completed calls that fell into the MOS result ranges defined for the assessment.

### Factors Affecting Call Quality by Call Script

The chart below breaks out call quality impairment factors to show the impact of each impairment (in points subtracted from the MOS) on the overall call quality per call script.

## Availability Analysis

### Explanation

Data may be unavailable due to five factors: endpoint unavailable, test timeout, Assessor unavailable, clock synchronization errors, and network connectivity problems.  Of all calls made, 2% were unavailable.  The charts below show the availability factors as percentages of the total unavailability.

### Availability Summary

### Availability by Day

### Availability by Hour

## All Groups Call Quality Summary

| Call Group | MOS |
| --- | --- |
| Raleigh_Bldg1-Raleigh_Bldg2 : G.711u (64 kbps) | 4.38 |
| SanJose-Bellevue : G.711u (64 kbps) | 4.33 |
| Raleigh_Bldg1-SanJose : G.711u (64 kbps) | 3.99 |
| Raleigh_Bldg1-Raleigh_Bldg2 : G.723.1-ACELP (5.3 kbps) | 3.64 |
| SanJose-Bellevue : G.723.1-ACELP (5.3 kbps) | 3.57 |
| Raleigh_Bldg1-SanJose : G.723.1-ACELP (5.3 kbps) | 3.36 |

# Appendix A – VoIP Quality Configuration

### Call Groups

| Endpoint 1 | Endpoint 2 | Call Script | Concurrent Calls | Port Range | Comment |
| --- | --- | --- | --- | --- | --- |
| Raleigh_Bldg1 | Raleigh_Bldg2 | G.711u (64 kbps) | 6 |  | G.711u (64 kbps) : Raleigh_Bldg1-Raleigh_Bldg2 6 |
| Raleigh_Bldg1 | Raleigh_Bldg2 | G.723.1-ACELP (5.3 kbps) | 6 |  | G.723.1-ACELP (5.3 kbps) : Raleigh_Bldg1-Raleigh_Bldg2 6 |
| Raleigh_Bldg1 | SanJose | G.711u (64 kbps) | 2 |  | G.711u (64 kbps) : Raleigh_Bldg1-SanJose 2 |
| Raleigh_Bldg1 | SanJose | G.723.1-ACELP (5.3 kbps) | 2 |  | G.723.1-ACELP (5.3 kbps) : Raleigh_Bldg1-SanJose 2 |
| SanJose | Bellevue | G.711u (64 kbps) | 4 |  | G.711u (64 kbps) : SanJose-Bellevue 4 |
| SanJose | Bellevue | G.723.1-ACELP (5.3 kbps) | 4 |  | G.723.1-ACELP (5.3 kbps) : SanJose-Bellevue 4 |

### Background Traffic

| Endpoint 1 | Endpoint 2 | Configured Data Rate | Port Range | Initial Delay | Comment |
| --- | --- | --- | --- | --- | --- |
| Raleigh_Bldg2 | Raleigh_Bldg1 | 10 Mbps (Ethernet) |  | 0 | 10 Mpbs (Ethernet) : Raleigh_Bldg2-Raleigh_Bldg1 |
| SanJose | Bellevue | 1.544 Mbps (T1) |  | 0 | 1.544Mbps (T1) : SanJose-Bellevue |

### Endpoints

| Name | Address | Increment | Setup Address | NAT Address | Version | Build Level | Operating System |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bellevue | 10.50.1.31 | n/a | 10.50.1.31 |  | 4.5 | 2169 | Windows 2000 |
| Raleigh_Bldg1 | 10.41.6.165 | n/a | 10.41.6.165 |  | 4.5 | 2169 | Sun Solaris |
| Raleigh_Bldg2 | 10.41.3.20 | n/a | 10.41.3.20 |  | 4.5 | 2169 | Linux |
| SanJose | 10.5.101.137 | n/a | 10.5.101.137 |  | 4.5 | 2169 | Windows XP (32-bit) |

### Call Scripts

| G.711u (64 kbps) |  |
| --- | --- |
| Codec | G.711u |
| Packet Loss Concealment | Off |
| Silence Suppression | Disabled |
| Voice Activity Rate | 50% |
| Delay Between Datagrams | 20 ms |
| Service Quality Name | n/a |
| Service Quality Type | n/a |
| Service Quality Value | n/a |
| Jitter Buffer | 40 ms |
| Additional Fixed Delay | 0 ms |
| Initial Delay | 0 |

| G.723.1-ACELP (5.3 kbps) |  |
| --- | --- |
| Codec | G.723.1-ACELP |
| Packet Loss Concealment | Off |
| Silence Suppression | Disabled |
| Voice Activity Rate | 50% |
| Delay Between Datagrams | 30 ms |
| Service Quality Name | n/a |
| Service Quality Type | n/a |
| Service Quality Value | n/a |
| Jitter Buffer | 60 ms |
| Additional Fixed Delay | 0 ms |
| Initial Delay | 0 |

### Assessment Options

| Schedule Type | Series of scheduled calls. |
| --- | --- |
| Call Duration | 1 minute |
| Schedule Duration | 2 days 0 hours |
| Call Interval | 60 minutes |
| Destination Port Minimum | Auto |
| Destination Port Maximum | Auto |
| Override Timing Record Duration | Off |
| Timing Record Duration | 5 seconds |
| Reporting Port | Auto |
| MOS Good | At least 4.03 MOS |
| MOS Acceptable | At least 3.60 MOS |
| MOS Poor | Any lower value |
| Lost Data Good | Less than or equal to 0.50% |
| Lost Data Acceptable | Less than or equal to 1.00% |
| Lost Data Poor | Any higher value |
| Jitter Buffer Loss Good | Less than or equal to 0.50% |
| Jitter Buffer Loss Acceptable | Less than or equal to 1.00% |
| Jitter Buffer Loss Poor | Any higher value |
| Delay Good | Less than or equal to 150 ms |
| Delay Acceptable | Less than or equal to 400 ms |
| Delay Poor | Any higher value |

### VoIP Quality - Design

# Appendix B – Call Quality Score Calculation

NetIQ uses a modified version of the ITU G.107 standard E-Model equation to calculate a Mean Opinion Score (MOS) estimate for each call group.

The E-Model, developed by the European Telecommunications Standards Institute (ETSI), has become ITU standard G.107. This algorithm is used to evaluate the quality of a transmission by factoring in the "mouth-to-ear" characteristics of a speech path.

In calculating the MOS score, Vivinet Assessor modifies the E-model slightly, using the following factors to calculate the MOS:

##### Delay

Also known as latency, delay is the sum of network delay, packetization delay, and jitter buffer delay. Delayed datagrams are perhaps the single greatest hindrance to VoIP call quality.

##### Jitter Buffer Loss

Jitter occurs when there are variations in datagram arrival times within a single transmission. When jitter exceeds jitter buffer capacity, datagram loss occurs, reducing call clarity.

##### Lost Data

Datagrams that never arrived at the receiving endpoint. A lost datagram can mean the loss of an entire word or syllable in a conversation.

A MOS of 5 is excellent; a MOS of 1 is unacceptably poor. The following table (taken from ITU G.107) summarizes the relationship between the MOS and user satisfaction:

| Mean Opinion Score (lower limit) | User Satisfaction |
| --- | --- |
| 4.34 | Very satisfied |
| 4.03 | Satisfied |
| 3.60 | Some users dissatisfied |
| 3.10 | Many users dissatisfied |
| 2.58 | Nearly all users dissatisfied |

By default, Vivinet Assessor maps MOS estimates to three quality categories:

| Mean Opinion Score Range | Quality | Meaning | Color |
| --- | --- | --- | --- |
| 5.0 - 4.03 | Good | Most or nearly all users satisfied | Green |
| 4.02 – 3.60 | Acceptable | Some users satisfied | Yellow |
| Below 3.60 | Poor | Most or nearly all users dissatisfied | Red |

# Appendix C – Codec Types and Values

In a VoIP transmission, the codec samples the sound and determines the data rate.  VoIP Quality assessments can be performed using different codec types to compare performance metrics.  See the Vivinet Assessor Help for more information.

##### G.711u

ITU standard for H.323-compliant codecs. Uses the u-law for companding, the most frequently used method in the USA. PLC is an option.

##### G.711a

ITU standard for H.323-compliant codecs. Uses the A-law for companding, a popular standard in Europe. PLC is an option.

##### G.726

A waveform coder that uses Adaptive Differential Pulse Code Modulation (ADPCM). ADPCM is a variation of pulse code modulation (PCM), which only sends the difference between two adjacent samples, producing a lower bit rate.

##### G.729

High-performing codec; offers compression with high quality.

##### G.729A

Also known as G.729 Annex A. High-performing codec; offers compression with high quality. Similar to G.729, but optimized for voice over frame relay, teleconferencing, and other applications.

G.723.1--MPMLQ

Uses the multipulse maximum likelihood quantization (MPMLQ) compression algorithm.

##### G.723.1--ACELP

Uses the conjugate structure algebraic code excited linear predictive compression (ACELP) algorithm.

In the table below, the "packetization delay" refers to the delay this codec introduces as it converts a signal from analog to digital; this delay is included in the MOS estimate, as is the "jitter buffer delay," the delay introduced by the effects of buffering to reduce delay variability. See the Vivinet Assessor Help for more information.

| Codec | Default Data Rate | Default Datagram Size | Packetization Delay | Default Jitter Buffer Delay | Theoretical Maximum MOS |
| --- | --- | --- | --- | --- | --- |
| G.711u; G.711a | 64 kbps | 20 ms | 1.0 ms | 2 datagrams (40 ms) | 4.40 |
| G.726 | 32 kbps | 20 ms | 1.0 ms | 2 datagrams 40 ms | 4.22 |
| G.729; G.729A | 8 kbps | 20 ms | 35.0 ms | 2 datagrams (40 ms) | 4.07 |
| G.723.1 MPMLQ | 6.3 kbps | 30 ms | 67.5 ms | 2 datagrams (60 ms) | 3.87 |
| G.723.1 ACELP | 5.3 kbps | 30 ms | 67.5 ms | 2 datagrams (60 ms) | 3.69 |

Note: The smallest jitter buffer allowed is 10 ms; this size is used to calculate the theoretical maximum MOS.

The figures for average revenue lost were calculated by TeleChoice, Inc., a leading telecom consulting and analyst firm.  The figures were derived based on specific and factual industry information provided by select U.S. companies in each segment as well as from U.S. government compiled information. Information was collected and analyzed in 2002 although much of the underlying data was from 2000 and 2001.

Assumes standard 8-hour workday.

Assumes standard 40-hour workweek.